NGA SV/MS Clock State File

Table 7 defines the content and format of the NGA SV/MS Clock State file, denoted as CLKyddd.txt in Table 3. The NGA SV/MS Clock State file is an ASCII text file containing a sequence of ascending time-ordered data records. There are redundant SV clock estimates in the NGA SV State and Covariance and NGA Clock SV/MS state datasets. The MS clock states contained in the NGA SV/MS Clock dataset will be used to populate the MS clock data of the Kalman Filter State Entries table in the Kalman filter.

Table 1. NGA SV/MS Clock State File Format

| COLUMNS | FORMAT | DESCRIPTION | UNITS | RANGE |
| --- | --- | --- | --- | --- |
| 1-4 | I4 | YEAR | YEARS |  |
| 5-7 | I3 | DAY OF YEAR (JAN 1 = 1) | DAYS | 1 - 366 |
| 8-15 | F8.2 | SECONDS INTO DAY | SECS | 0 - 86399 |
| 16-17 | I2 | SATELLITE OR STATION (0=SATELLITE,1=STATION) |  | 0 - 1 |
| 18-23 | I6 | FOR SATELLITE RECORD (1- 32): SATELLITE PRN FOR STATON RECORD (> 32): NGA MS REFERENCE NUMBER |  |  |
| 24 | A1 | INFORMATION TYPE (N=NOMINAL CLOCK, C=COMPOSITE CLOCK, E=EVENT INFORMATION |  | N, C, E |
| 25-27 | A3 | EVENT NAME FOR INFORMATION TYPE = ‘N’ OR ‘C’, FIELD IS BLANK FOR INFORMATION TYPE = ‘E’ FIELD CONTAINS TWO- LETTER CODE AS FOLLOWS: ZC: Z-COUNT ADJUST (SV) CF: C-FIELD ADJUST (SV) RE: REINITIALIZEATION (SV/MS) FC: |  |  |
| COLUMNS | FORMAT | DESCRIPTION | UNITS | RANGE |
|  |  | FREQUENCY CHANGE (SV/MS) TC: TIME CHANGE (MS) |  |  |
| 28-48 | E21.14 | BIAS | NSEC | float |
| 49-63 | E15.8 | DRIFT | PSEC/SEC | float |
| 64-78 | E15.8 | DRIFT RATE | PSEC/SEC/DAY | float |
| 79-86 | F8.4 | COMPOSITE CLOCK WEIGHT |  | float |

Notes:

Figure 4 shows an example of the NGA SV/MS Clock State data.

Figure 1. NGA SV/MS Clock State Data Example

2019021    0.00 0    10C    1.32045233631681D+05-6.69258604D+00-3.40988289D-02 10.0152

2019021    0.00 0    11C   -6.72444202934369D+05 5.43368193D+00 2.09694114D-02 10.0120

2019021    0.00 0    12C    2.74706049630299D+05 3.10123934D+00-3.32055881D-03 00.0000

2019021    0.00 0    13C   -7.76255118535666D+04 1.32719045D+00 4.95727081D-03 00.0000

2019021    0.00 0    14C   -9.28210258362750D+04 6.36440847D-01 3.32050485D-03 00.0000

2019021    0.00 0    15C   -3.31393024226380D+05 1.59706117D+00 4.00420738D-03 10.0152

2019021    0.00 0    16C   -1.40004559016747D+04-2.37157751D+00-5.05201949D-03 10.0187

2019021    0.00 0    17C   -1.86126634848664D+03 5.42887601D+00 9.90062454D-03 00.0000

2019021    0.00 0    18C    1.78232345114387D+04 3.81790667D+00-1.11630995D-03 10.0132

2019021    0.00 0    19C   -3.61207419088012D+05 4.02716221D+00 4.03989506D-03 10.0143

2019021    0.00 0     1C   -1.48834851442695D+05 6.50450959D+00-1.72924382D-02 00.0000

2019021    0.00 0    20C    5.21343619078594D+05 4.22941660D-01-1.13561884D-03 10.0167

2019021    0.00 0    21C   -2.65382740302793D+05 6.99469302D+00 8.20426237D-03 00.0000

2019021    0.00 0    22C   -6.27341575654231D+05 8.57134824D+00 1.90333839D-02 10.0127

2019021    0.00 0    23C   -1.99887372068137D+05 1.21463854D+00 3.96953221D-03 10.0175

2019021    0.00 0    24C   -6.10671306479986D+04 5.00569841D-01 0.00000000D+00 10.0077

2019021    0.00 0    25C   -7.08818724236854D+05 5.42423952D+00 2.32717629D-03 10.0150

2019021    0.00 0    26C    1.02045297583900D+05 1.05665238D+01-7.07732484D-03 10.0156

2019021    0.00 0    27C   -5.26013835067751D+04 6.73017045D+00-2.34497330D-02 10.0195

2019021    0.00 0    28C    7.62530499494371D+05 1.10392365D+00-8.62142262D-03 10.0132

2019021    0.00 0    29C    2.88797986317761D+05 8.46423026D+00-9.20362898D-03 00.0000

2019021    0.00 0     2C   -1.18496791145265D+05 1.00580384D+01 7.33029441D-03 00.0000

2019021    0.00 0    30C   -3.92016694795718D+04 6.19756128D+00-8.95804923D-03 10.0159

2019021    0.00 0    31C    6.43453450875975D+04 2.64477029D+00 6.09444072D-04 10.0146

2019021    0.00 0    32C   -2.65517536953511D+05 1.74551742D+01 4.98163903D-02 00.0000

2019021    0.00 0     3C    1.78482693799851D+05 2.36090617D+00-2.32156511D-02 00.0000

2019021    0.00 0     4C   -1.43491241608455D+05 1.08370887D+01 1.76268514D-01 00.0000

2019021    0.00 0     5C    8.76555812768078D+02 1.33612786D-01-1.54344628D-03 10.0142

2019021    0.00 0     6C    2.92788476958023D+05 7.70127000D+00-2.06880524D-02 10.0158

2019021    0.00 1 85128C    3.18019860545206D+03 9.24085051D-03 0.00000000D+00 10.0682

2019021    0.00 1 85129C   -2.23752312769859D+04-4.61486467D-02 0.00000000D+00 10.0487

2019021    0.00 1 85130C    3.46249026240624D+03-1.93562926D-01 0.00000000D+00 10.0340

2019021    0.00 1 85131C    5.51286115929680D+03 1.02117390D-01 0.00000000D+00 10.0387

2019021    0.00 1 85402C   -1.45896367724536D+03-1.10925302D-01 0.00000000D+00 10.0336

2019021    0.00 1 85403C   -4.36720502756103D+03-1.12935319D-01 0.00000000D+00 10.0410